FEDERAL EMERGENCY MANAGEMENT AGENCY

O.M.B. No. 3067-0077

Expires December 31, 2005

Section B: Items (B1-12) must always be completed except as described in B9 for Zone A (without a BFE).

### EC QUICK REFERENCE GUIDE FOR CRS COMMUNITIES

#### Details for Completing an Elevation Certificate are in the Instructions– See Note on Page 2

Version Date: March 2003

NATIONAL FLOOD INSURANCE PROGRAM

ELEVATION CERTIFICATE

B6 – FIRM Index Date.  Enter the date from the latest FIRM INDEX where the building is located, not the FIRM panel date except as noted.  NOTE:  For communities with only one FIRM panel there is no FIRM Index, use the panel date (same as B7).  You may check the NFIP Community Status Book to ensure that you are using the current FIRM Index and current panel for the property location.  This may be done on the www.fema.gov website or by calling 1-800-427-4661.  Due to recent incorporations or annexations, the correct FIRM Index may be different than the one for the community entered in B1.

Important:  Read the instructions on pages 1 - 7.

Section A: 911 Address of building location, if possible.  Complete all applicable items.  Only the Latitude/Longitude item is optional.  Attach a map or sketch or provide directions in Section D comments on Page 2, if necessary.

| ## SECTION A - PROPERTY OWNER INFORMATION | For Insurance Company Use: |
| --- | --- |
| # BUILDING OWNER’S NAME | Policy Number |
| BUILDING STREET ADDRESS (Including Apt., Unit, Suite, and/or Bldg. No.) OR P.O. ROUTE AND BOX NO. B1 – Enter the NFIP name and number (six digits) of the community where the building is located. For a building that is in area that has been annexed by one community but is shown on another community’s FIRM, enter the community name and six digit number of the annexing community. For buildings located in unincorporated areas of a parish, enter the NFIP name and number for the parish, not the city listed in the postal address. A countywide FIRM number (5-digits plus “C”) is not an NFIP community number | Company NAIC Number |
| CITY STATE ZIP CODE |  |
| PROPERTY DESCRIPTION (Lot and Block Numbers, Tax Parcel Number, Legal Description, etc.) B7 - FIRM PANEL date. Enter the date from the FIRM panel where the building is located. The panel date may be the same or earlier than the FIRM Index date but not later. Due to annexations, the correct FIRM panel may be different than the one for the community entered in B1. |  |
| BUILDING USE (e.g., Residential, Non-residential, Addition, Accessory, etc. Use a Comments area, if necessary.) |  |
| LATITUDE/LONGITUDE (OPTIONAL) HORIZONTAL DATUM: SOURCE: GPS (Type): B8 – Flood Zone(s). Enter all flood zones in which the building is either partially or totally located. ( ##o - ##’ - ##.##” or ##.#####o) NAD 1927 NAD 1983 USGS Quad Map Other: |  |
| SECTION B - FLOOD INSURANCE RATE MAP (FIRM) INFORMATION B4 – Enter the FIRM map and panel number where the building is located. Due to annexations, this FIRM may be a different than the one for the community entered in B1. For FIRM panel not in parish wide format: use the 10-digit (6-digit community and 4-digit panel) number shown on the map. For FIRM panel in parish wide format: use the parish wide number (5 digits plus “C”) followed by the 4-digit panel number shown on the map. For older FIRMs where the panel number is not four digits add enough zeros in front of the number to make it four digits. Map/Panel Number does not include the Suffix letter following the panel number – Enter Suffix letter in Item B5. |  |

| B1. NFIP COMMUNITY NAME & COMMUNITY NUMBER B9 – BFE. Enter BFE determined from the Flood Insurance Study Profile (preferred source), Flood Elevation Table, FIRM or other source. A BFE should be provided for each flood zone listed in Item B8. Remember to enter depth for AO Zones. Leave blank if unnumbered Zone A (and a BFE has not been determined). | B2. COUNTY NAME | B3. STATE |
| --- | --- | --- |

| B4. MAP AND PANEL NUMBER | B5. SUFFIX | B6. FIRM INDEX DATE | B7. FIRM PANEL EFFECTIVE/REVISED DATE | B8. FLOOD ZONE(S) | B9. BASE FLOOD ELEVATION(S) (Zone AO, use depth of flooding) |
| --- | --- | --- | --- | --- | --- |

B10. Indicate the source of the Base Flood Elevation (BFE) data or base flood depth entered in B9.

B10 – Check appropriate square for source(s) of BFE determination indicated in B9.  “Other” would be Corps, USGS, etc.  “Community” or “Other” normally would be the source used to establish BFE for unnumbered A Zones.  For developments of more than 50 lots or 5 acres, establishment of BFE’s is required by the community’s floodplain management ordinance.

FIS Profile	 FIRM	 Community Determined	 Other (Describe):

B11. Indicate the elevation datum used for the BFE in B9:  NGVD 1929	 NAVD 1988	 Other (Describe):

B12. Is the building located in a Coastal Barrier Resources System (CBRS) area or Otherwise Protected Area (OPA)?	 Yes	  No	 Designation Date

SECTION C - BUILDING ELEVATION INFORMATION (SURVEY REQUIRED) C1 – A post-construction Elevation Certificate is required when construction is completed for floodplain management purposes and insurance rating purposes. Select “Finished Construction” which means all machinery and/or equipment such as furnaces, hot water heaters, heat pumps, air conditioners, and elevators and their associated equipment have been installed and the grading around the building is completed.

C1. Building elevations are based on:  Construction Drawings*		 Building Under Construction*		 Finished Construction

*A new Elevation Certificate will be required when construction of the building is complete.

B11 and B12 – Check appropriate boxes.

C2. Building Diagram Number _  (Select the building diagram most similar to the building for which this certificate is being completed - see pages 6 and 7.  If no diagram

accurately represents the building, provide a sketch or photograph.)

C3. Elevations – Zones A1-A30, AE, AH, A (with BFE), VE, V1-V30, V (with BFE), AR, AR/A, AR/AE, AR/A1-A30, AR/AH, AR/AO

Section C: Complete for all zones except AO and A (without a BFE) Zones (unless EC is to be used for LOMA or LOMR-F).  Enter elevations for Items C3.a-g to the nearest tenth of a foot.

Complete Items C3.-a-i below according to the building diagram specified in Item C2. State the datum used. If the datum is different from the datum used for the BFE in

C2 – Enter appropriate diagram number (1 through 8) based on building type, as illustrated on Pages 6 and 7 of the Instructions.   If unsure of the correct diagram number, select the best option or provide a sketch or photograph of building and enter all elevations in C3.a-g.  NOTE:  Always completed.

Section B, convert the datum to that used for the BFE. Show field measurements and datum conversion calculation.  Use the space provided or the Comments area of

Section D or Section G, as appropriate, to document the datum conversion.

Datum         Conversion/Comments

C3 – Confirm the elevation datum source and any necessary conversions, then complete C3.a-i according to the instructions for the diagram number selected in C2.  See illustrations on Pages 6 and 7 of the Instructions.

License Number, Embossed Seal,

Signature, and Date

Elevation reference mark used       Does the elevation reference mark used appear on the FIRM?    Yes    No

  a) Top of bottom floor (including basement or enclosure)		              .     ft.(m)

  b) Top of next higher floor				               .    ft.(m)n

C3.a – Elevation of top of lowest floor.  This should never be the elevation of the attached garage slab (See C3.d).  NOTE:  Always completed.

  c) Bottom of lowest horizontal structural member (V zones only)	               .    ft.(m)

C3.c – Enter elevation only for V Zone buildings.  Otherwise, enter N/A.

  d) Attached garage (top of slab)				              .     ft.(m)

  e) Lowest elevation of machinery and/or equipment

C3.b – Complete only if the building has two floor levels (not including the attached garage) as shown in the diagram illustrations on pages 6 and 7 of the Instructions.  Otherwise, enter N/A.

servicing the building (Describe in a Comments area)		               .    ft.(m)

C3.d – Enter elevation of the top of the attached garage slab.   If none, enter N/A.

  f) Lowest adjacent (finished) grade (LAG)			               .    ft.(m)

  g) Highest adjacent (finished) grade (HAG)			              .     ft.(m)

C3.g – Enter elevation of finished highest adjacent grade (ground, sidewalk, patio slab or deck support immediately next to the building). For Zone AO, use natural grade, if available.  Measure to nearest tenth of a foot if certificate is used for LOMA or LOMR-F request.  NOTE:  Always completed.

  h) No. of permanent openings (flood vents) within 1 ft. above adjacent grade

  i) Total area of all permanent openings (flood vents) in C3.h      sq. in. (sq. cm)

C3.e – Enter the lowest elevation of the equipment servicing the building.  If installed, servicing equipment elevation must be entered for the certificate to be based on Finished Construction.  Describe equipment in comments.  Enter N/A only if servicing equipment will not be installed.

## SECTION D - SURVEYOR, ENGINEER, OR ARCHITECT CERTIFICATION

This certification is to be signed and sealed by a land surveyor, engineer, or architect authorized by law to certify elevation information. C3.h – Number of permanent openings that are no higher than 1.0 foot above the adjacent grade. If none, enter 0. C3.f – Enter elevation of finished lowest adjacent grade (ground, sidewalk, patio slab or deck support immediately next to the building. For AO Zone, use natural grade, if available. Measure to nearest tenth of a foot if certificate is used for a LOMA or LOMR-F. Note: Always completed. I certify that the information in Sections A, B, and C on this certificate represents my best efforts to interpret the data available. I understand that any false statement may be punishable by fine or imprisonment under 18 U.S. Code, Section 1001.

CERTIFIER’S NAME LICENSE NUMBER C3.i – Total area of permanent openings as described in C3.h. If none, enter 0.

ADDRESS CITY STATE ZIP CODE Place signature and embossed seal in box to certify elevations. Flat stamp is authorized only in states that do not authorize embossed seal over the signature of a professional.

SIGNATURE DATE TELEPHONE Section D: Complete as indicated by person authorized to certify elevations.

| IMPORTANT: In these spaces, copy the corresponding information from Section A. Note: The comments in this guide do not supercede the FEMA Instructions and are not intended to cover all situations when completing an elevation certificate. Please refer to the FEMA/NFIP Elevation Certificate and Instructions for full details. | For Insurance Company Use: |
| --- | --- |
| BUILDING STREET ADDRESS (Including Apt., Unit, Suite, and/or Bldg. No.) OR P.O. ROUTE AND BOX NO. Repeat address information from Section A in order to correctly match pages 1 and 2. | Policy Number |
| CITY STATE ZIP CODE Section D (continued): Use comments section to provide datum, elevation or other relevant information not specified on the front. Copy both sides of the certificate for the community official, insurance agent and building owner. When an elevation for item C3.e is entered, describe machinery/equipment type here. | Company NAIC Number |
| ## SECTION D - SURVEYOR, ENGINEER, OR ARCHITECT CERTIFICATION (CONTINUED) |  |
| Copy both sides of this Elevation Certificate for (1) community official, (2) insurance agent/company, and (3) building owner. |  |
| COMMENTS |  |
| Section E: Use only for AO or Zone A (without a BFE) and certificate is not used to support information for a LOMA or LOMR-F. Otherwise, complete Section C instead. |  |
| Check here if attachments E2 - This corresponds to the elevation of “a” in the illustration of the diagram number selected for E1. Indicate how much this floor is above or below the highest adjacent grade. |  |
| ## SECTION E - BUILDING ELEVATION INFORMATION (SURVEY NOT REQUIRED) FOR ZONE AO AND ZONE A (WITHOUT BFE) |  |
| For Zone AO and Zone A (without BFE), complete Items E1 through E4. If the Elevation Certificate is intended for use as supporting information for a LOMA or LOMR-F, Section C must be completed. Section G: The community official who is authorized by law or ordinance to administer the community’s floodplain management ordinance can complete Sections A, B, C (or E), and G of this Elevation Certificate. If the authorized community official completes Sections C, E, or G, complete the appropriate item(s) and sign this section. Check Item G1. If Section C is completed with elevation data from other documentation, including elevations obtained from the Community Rating System Elevation Software, that has been signed and embossed by a licensed surveyor, engineer, or architect who is authorized by law to certify elevation information. Indicate the source of the elevation data and the date obtained in the Comments area of Section G. If you are both a community official and a licensed land surveyor, engineer, or architect authorized by law to certify elevation information, and you performed the actual survey for a building in Zones A1-A30, AE, AH, A (with BFE), VE, V1-V30, V (with BFE), AR, AR/A, AR/A1-A30, AR/AE, AR/AH, or AR/AO, you must also complete Section D. Check Item G2. If information is entered in Section E by the community for a building in Zone A (without a FEMA-issued or community-issued BFE) or Zone AO. Check Item G3. If the information in Items G4.-G9. has been completed for community floodplain management purposes to document the as-built lowest floor elevation of the building. Section C of the Elevation Certificate records the elevation of various building components but does not determine the lowest floor of the building or whether the building, as constructed, complies with the community’s floodplain management ordinance. This must be done by the community. E3 - This corresponds to the elevation of “b” in the illustration of the diagram number selected for E1. Enter how much this floor is above the highest adjacent grade. Enclosures below the elevated floor (as shown in the illustrations for building diagrams 6, 7 and 8) require openings. Items C3.h and C3.i must also be completed for these diagram numbers. E1 - Enter appropriate diagram number (1 through 8) based on building type, as illustrated on Pages 6 and 7 of the Instructions. E5: - Community official must determine if building in Zone AO was constructed according to the community’s floodplain management ordinance. E1. Building Diagram Number (Select the building diagram most similar to the building for which this certificate is being completed – see pages 6 and 7. If no diagram accurately represents the building, provide a sketch or photograph.) E2. The top of the bottom floor (including basement or enclosure) of the building is ft.(m) in.(cm) above or below (check one) the highest adjacent grade. (Use natural grade, if available). E3. For Building Diagrams 6-8 with openings (see page 7), the next higher floor or elevated floor (elevation b) of the building is ft.(m) in.(cm) above the highest adjacent grade. Complete items C3.h and C3.i on front of form. E4. The top of the platform of machinery and/or equipment servicing the building is __ ft(m) __ in(cm) above or below (check one) the highest adjacent grade. (Use natural grade if available) E4 – Enter the lowest elevation of the equipment servicing the building. Describe equipment in comments in Section F. Enter N/A only if servicing equipment will not be installed. E5. For Zone AO only: If no flood depth number is available, is the top of the bottom floor elevated in accordance with the community’s floodplain management ordinance? Yes No Unknown. The local official must certify this information in Section G. |  |
| # SECTION F - PROPERTY OWNER (OR OWNER’S REPRESENTATIVE) CERTIFICATION |  |
| The property owner or owner’s authorized representative who completes Sections A, B, C (Items C3.h and C3.i only), and E for Zone A (without a FEMA-issued or community-issued BFE) or Zone AO must sign here. The statements in Sections A, B, C, and E are correct to the best of my knowledge. Section F: Certification of property owner or owner’s representative who completes Sections A, B, C (Items c3.h and c3.i only) and E for Zone AO and Zone A (without a BFE). The address indicated must be the actual mailing address of the property owner or representative who provided the information on the certificate. If a community official completes Section E, certification is done in Section G. |  |
| PROPERTY OWNER’S OR OWNER’S AUTHORIZED REPRESENTATIVE’S NAME |  |
| ADDRESS CITY STATE ZIP CODE |  |
| SIGNATURE DATE TELEPHONE |  |
| COMMENTS |  |
| # Check here if attachments |  |
| # SECTION G - COMMUNITY INFORMATION (OPTIONAL) |  |

The local official who is authorized by law or ordinance to administer the community’s floodplain management ordinance can complete

Sections A, B, C (or E), and G of this Elevation Certificate.  Complete the applicable item(s) and sign below.

G1.  The information in Section C was taken from other documentation that has been signed and embossed by a licensed surveyor,

engineer, or architect who is authorized by state or local law to certify elevation information.  (Indicate the source and date of the

elevation data in the Comments area below.)

G2.  A community official completed Section E for a building located in Zone A (without a FEMA-issued or community-issued BFE) or Zone AO.

G3.  The following information (Items G4-G9) is provided for community floodplain management purposes.

| G4. Permit Number G4 - 9 – Complete as indicated to determine if a structure has been built in accordance with your community’s floodplain management ordinance. | # G5. Date Permit Issued | # G6. DATE CERTIFICATE OF COMPLIANCE/OCCUPANCY ISSUED |
| --- | --- | --- |

G7. This permit has been issued for:	 New Construction	 Substantial Improvement

G8. Elevation of as-built lowest floor (including basement) of the building is:	    .   ft.(m)	Datum:

G9. BFE or (in Zone AO) depth of flooding at the building site is:	     .    ft.(m)	Datum:

Enter all the listed information for the community official.  Official must sign and date the certificate.

LOCAL OFFICIAL’S NAME TITLE

COMMUNITY NAME TELEPHONE

SIGNATURE DATE Enter comments as necessary. When items G4-G9 are completed, the local official can use the comment section to document building compliance for building features such as machinery/equipment and openings.

COMMENTS

Version Date: March 2003

Check here if attachments